Methodology

This section describes how we compare lead-acid and lithium-ion batteries for informal e-mobility in Bangladesh. The analysis combines a total cost of ownership (TCO) model that includes scrap value, a temperature-dependent degradation and lifetime model, and a set of operating and policy scenarios.

3.1. Study System Boundary and Functional Unit

The study focuses on the informal e-mobility sector in Bangladesh, specifically "easybikes" (small three-wheelers). To ensure comparability with international literature, the functional unit is defined as 1 km of passenger transport and 1 kWh of nominal battery capacity. We compare traditional deep-cycle Lead-Acid (LA) batteries with Lithium-Iron-Phosphate (LFP) Li-ion batteries.

3.2. Integrated Thermal-Chemical Degradation Model

Drawing from the need for real-world environmental modeling highlighted by Khaleel et al., we employ an Arrhenius-based degradation model to simulate the 30–40 °C tropical climate.

- Capacity Fade Equation: We model the State of Health (SoH) as a function of temperature T and energy throughput E tp where Ea is the activation energy and R is the gas constant.

- SoH=f(Etp​)⋅exp(-EaR.T​​)

- End-of-Life (EoL) Threshold: Consistent with EV standards, LFP batteries are retired at 80% SoH. For Lead-Acid, EoL is defined by a combination of capacity loss and internal resistance growth that prevents a full day's operation.

3.3. Scrap-Value-Aware Total Cost of Ownership (TCO) Model

To quantify the "Lead-Acid Trap," we develop a TCO model that includes discounted cash flows over an 8-year horizon. Unlike standard models, we explicitly endogenize the scrap value (Sv)

where Cp is the purchase price, $r$ is the discount rate for low-income operators, and (Sv)for Lead-Acid is set at 40–50% of Cp

3.4. Prospective Policy Scenario Design

We simulate four scenarios (2025–2035) using the prospective modeling approach of Arrieta et al.:

- Baseline: Current informal market (High LA scrap value, high Li-ion duty).

- Duty Reform: Reduced customs duty on Li-ion components (referencing Bangladesh's 2025 policy).

- The "Lithium Rebate": A policy-driven buyback scheme for LFP to create a residual value where none currently exists.

- Circular Extension (Second-Life): Repurposing retired LFP packs for stationary solar-coupled DC microgrids.

4. Results and Discussion

4.1. Breaking the "Lead-Acid Trap"

The baseline TCO analysis reveals that Lead-Acid remains cheaper in the short term due to the 40–50% liquidity returned at end-of-life. However, once thermal degradation at 35 °C is modeled, the replacement frequency of LA is 3x higher than LFP. The breakeven point for LFP occurs at Year 3 under the "Duty Reform" scenario.

4.2. Impact of High Usage Intensity

For high-mileage operators (full-day shifts), LFP achieves a 30% lower cost per km compared to LA. In contrast, for low-usage "peak-only" operators, the "Lead-Acid Trap" remains dominant as the capital cost of LFP is not recovered fast enough.

4.3. Second-Life Value Creation

By extending the LFP lifecycle into a second stationary application (e.g., community PV storage), the effective environmental impact is reduced by nearly 49%, and the TCO is further lowered by the potential resale value of the retired EV pack to microgrid integrators.