Khulna University of Engineering & Technology (KUET)

Department of Electrical and Electronic Engineering

A Bangladesh-Specific Techno-Economic and Degradation-Based Comparison of Lead-Acid, NMC, and LFP Batteries for Electric Easy-Bikes

A Research Proposal Submitted for Master’s of Science and Engineering

Submitted By

Razowan Kabir

Student ID: 2603504

Session: Jan-2026

Department of Electrical and Electronic Engineering

Khulna University of Engineering & Technology

Submitted To

Professor Dr. Noruttam Kumar Roy

Department of Electrical and Electronic Engineering

Khulna University of Engineering & Technology

05 September 2026

APPROVAL PAGE

The research proposal titled “When Does Lithium Replace Lead-Acid? A Bangladesh-Specific Techno-Economic and Degradation-Based Comparison of Lead-Acid, NMC, and LFP Batteries for Electric Easy-Bikes," submitted by Razowan Kabir (Student ID: 2603504), has been examined and is hereby recommended for approval and acceptance in partial fulfillment of the requirements for the degree of Master of Science in Electrical and Electronic Engineering.

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

Head of the

Department of EEE

Khulna University of Engineering and Technology.

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

Dr. Noruttam Kumar Roy

Supervisor

Department of EEE

Khulna University of Engineering and Technology.

A Bangladesh-Specific Techno-Economic and Degradation-Based Comparison of Lead-Acid, NMC, and LFP Batteries for Electric Easy-Bikes:

An overview of research objectives and methodology for Master’s-level study

# Introduction

Electric easy-bikes/e-rickshaws are a dominant informal-transport mode in Bangladesh and are almost universally lead-acid powered despite short battery life and low efficiency [5]–[8]. Lithium alternatives (NMC, LFP) are technically superior on several dimensions but cost more, and their suitability for Bangladesh's hot, high-utilization, informal operating context is unverified locally. This proposal develops a Bangladesh-specific techno-economic, degradation, energy, environmental, and policy framework comparing lead-acid, NMC, and LFP for easy-bikes, combining a 60–150 operator survey across three cities, secondary government data, a MATLAB energy–degradation–TCO model, break-even/sensitivity analysis, a simplified environmental comparison, and policy-scenario evaluation—scoped to be feasible in four months, with recommended (Monte Carlo, environmental, and policy) modules clearly separated from the essential core. The study is designed to determine, not assume, which technology is preferable and under what conditions.

# Background, Problem Statement, and Research Gap

Easy-bikes/e-rickshaws now number in the hundreds of thousands to over a million nationally [7], [16], almost all lead-acid powered for their low cost, established market, and resale/scrap value [2], [4], but suffering short life (often 6–12 months) from deep discharge, poor charging, and heat [2], [4], [8]. LFP and NMC lithium alternatives offer longer life and better thermal stability (LFP especially) at a higher upfront cost [4], [4],[17]; Bangladesh's electricity mix, tariff, informal ULAB recycling sector (>80% of recycled lead, largely unregulated and a major lead-exposure source) [13], and evolving EV policy [14]–[16] make this a genuinely local question, not one answerable by importing foreign figures.

Problem: simply asserting lithium is “better” is scientifically insufficient. The question is under what Bangladesh-specific operating, economic, environmental, and policy conditions each technology becomes preferable.

Research gap (provisional—to be confirmed in Phase 1): no identified study combines (a) a three-chemistry comparison for Bangladesh easy-bikes using primary field data, (b) temperature-linked degradation feeding a discounted, replacement-aware TCO, (c) lead-acid scrap value and informal recycling explicitly modeled, and (d) results connected to quantified policy thresholds. Related work exists for Indian e-rickshaws [2], [2],[4] and Indian two-wheelers [3], and for Bangladesh easy-bike operations pre-dating lithium adoption [5], [6], but not this combination.

## Main research question

Under Bangladesh's high-temperature, high-utilization easy-bike conditions, when do lead-acid, NMC, and LFP batteries become economically/environmentally preferable, and how do degradation, lifetime, electricity use, replacement frequency, scrap value, and policy affect this?

## Sub-questions (RQ1–RQ11, condensed)

Technical/operational characteristics of the three chemistries in this application (RQ1); degradation vs. usage/DoD/temperature (RQ2); charging energy and annual electricity use (RQ3); TCO drivers—price, lifetime, replacement, electricity, maintenance, scrap (RQ4); lowest 5-/10-year TCO (RQ5); break-even price/utilization conditions (RQ6); influence of lead-acid scrap value (RQ7); temperature effects on relative performance (RQ8); robustness to parameter uncertainty (RQ9); environmental performance under Bangladesh grid/EOL scenarios (RQ10); policy measures to accelerate the preferable technology (RQ11).

# Aim, Objectives, Hypotheses, Significance, Scope

Aim: to build a Bangladesh-specific techno-economic/degradation/energy/environmental/policy framework comparing the three technologies and identify the conditions under which each is preferable.

## Objectives

O1. Characterize the three technologies; O2. Collect Bangladesh operational/economic data (survey + secondary); O3. Quantify energy requirements; O4. Model temperature/usage-dependent degradation and lifetime; O5. Compute 5-/10-year discounted TCO incl. scrap value; O6. Break-even and sensitivity analysis; O7. Simplified environmental comparison; O8. Evaluate policy scenarios against results.

## Hypotheses (tested, not assumed)

H1. Lead-acid has a lower initial cost. H2. Lithium needs less grid electricity per useful energy delivered. H3. LFP outlasts lead-acid at comparable utilization. H4. Higher utilization favors long-life lithium. H5. Scrap value narrows lead-acid's cost disadvantage. H6. Temperature materially affects lifetime/TCO. H7. The preferred technology shifts with price, lifetime, utilization, tariff, temperature, and scrap. H8. NMC and LFP differ economically/environmentally despite both being Li-ion.

## Significance

Academic: integrates degradation, TCO, scrap value, and environmental analysis for an understudied vehicle class [3], [9]-[11]. Practical: evidence for operators/suppliers/manufacturers. Policy: quantified thresholds for Bangladesh's active EV incentive and informal-transport debate [14]-[16]. Societal: operator livelihoods and informal lead-recycling health impacts [7], [13].

## Scope

Bangladesh easy-bikes/e-rickshaws only, three named chemistries, operator-level economics, degradation, a simplified (not full-ISO) environmental comparison, and a small policy-scenario set. Excludes other EV categories and national-scale extrapolation unless a credible fleet source is found at low extra cost.

# Methodology (Essential Core)

## Survey & secondary data

60–150 operators, 3 cities (extendable to 4), piloted with 15–20 before the main round; variables cover operator/vehicle/battery/charging/usage/maintenance/end-of-life/adoption willingness. Secondary data (priority order): BBS, BERC, SREDA, relevant ministries, NBR, peer-reviewed literature, and manufacturer datasheets.

[DATA REQUIRED] Electricity tariff category/rate, seasonal temperature series, easy-bike fleet-size estimate, and grid emission factor are not yet sourced.

Figure 1: 1Methodology flowchart illustrating data collection, core MATLAB analytical modules, validation process, and policy analysis for the electric easy-bike battery assessment.

## Core equations

Eusable = V×Ah×DoD     Egrid =Eusableηbatt·ηcharger      Eannual = Ddaily×ekm×Ndays

SoHi = f(N,T,DoD,C)     Lifetime =NEOLcyclesday×daysyear

TCOi = Cinit+Creplace+Celec+Cmaint-Cscrap        NPV = ΣCt1+rt

Degradation uses literature-adapted temperature/DoD scenario multipliers (not an originally fitted Arrhenius model) [4], [12], [17], validated against survey-reported lifetimes. TCO follows standard discounted NPV formulations [10], [11]. Break-even computed pairwise (LA-NMC, LA-LFP, NMC-LFP); “no break-even within horizon” reported when true rather than forced.

## Sensitivity, environment, policy, stats

- Essential: one-at-a-time sensitivity (price, lifetime, temperature, distance, DoD, efficiency, tariff, scrap, maintenance, discount rate) as a tornado chart; descriptive statistics and group comparisons on survey data.

- Recommended (dropped first if time is short): Monte Carlo using literature-justified ranges [9] for P (lowest TCO); simplified lifecycle-oriented environmental comparison (manufacturing+operation+EOL, kg CO₂-eq/vehicle-km) reflecting Bangladesh's informal ULAB recycling [13]; 2–3 quantified policy scenarios (subsidy/duty) [14], [15].

## MATLAB & validation

Modular scripts (input→energy→degradation→lifetime→replacement→TCO→environment→break-even→sensitivity→[Monte Carlo]→stats→plots) using a battery. LA/NMC/LFP structure; coding begins only once real parameters are sourced. Energy, degradation, and TCO outputs validated against survey and literature benchmarks with explicit error reporting; gaps stated as limitations, not hidden.

# 16-Week Work Plan

Compressed from an original 24-week design by reducing the survey to 60–150 respondents/3 cities and marking Monte Carlo, environmental analysis, and policy scenarios “recommended” (first to be cut if time is short); national-scale analysis and full ISO-LCA are excluded from the outset.

| Week(s) | Phase | Key activities | Priority |
| --- | --- | --- | --- |
| 1–2 | Literature review | Systematic search, literature matrix, confirm/refine research gap | Essential |
| 2–3 | Proposal finalization | Finalize RQs, objectives, and survey instrument draft. | Essential |
| 3 | Ethics + secondary data | Submit ethics application; begin BBS/BERC/SREDA/NBR data collection. | Essential |
| 4 | Pilot survey | 15–20 operators; refine questionnaire | Essential |
| 5–7 | Main survey | 60–150 operators across 3 locations; continue secondary data | Essential |
| 7 | Data cleaning | Remove duplicates/outliers; document assumptions. | Essential |
| 8–9 | Statistical analysis | Descriptives, group comparisons, correlation | Essential |
| 8–10 | Energy + degradation model | MATLAB scripts 01–04, populated with sourced parameters | Essential |
| 10–11 | Lifetime, replacement, TCO | MATLAB scripts 05–06; 5-yr and 10-yr TCO | Essential |
| 11 | Validation | Compare model outputs to literature/survey benchmarks. | Essential |
| 12 | Break-even + sensitivity | MATLAB scripts 08–09; tornado chart | Essential |
| 13 | Monte Carlo (if time permits) | MATLAB script 10; literature-based ranges only | Recommended |
| 13 | Environmental analysis | Simplified manufacturing/operation/EOL comparison | Recommended |
| 14 | Policy scenarios | 2–3 quantified scenarios against TCO results | Recommended |
| 15 | Figures, tables, results write-up | Finalize Chapters 5–6 (Results, Discussion). | Essential |
| 16 | Full draft, internal review, revision | Complete proposal-to-report transition; supervisor review | Essential |

# Resources, Budget, Ethics, Limitations

Software

- MATLAB (core computation, all modeling scripts)

- MATLAB Statistics and Machine Learning Toolbox (survey statistics, Monte Carlo)

- Spreadsheet software (raw data handling, master parameter table)

- Reference manager (Zotero or Mendeley)

Field resources

- Printed/digital questionnaire, informed-consent forms, local transport for survey visits, mobile data-collection device (phone/tablet).

Budget

| Item | Estimated cost (BDT) | Note |
| --- | --- | --- |
| Local transport for survey (2 cities, ~3 visits each) | 8,000–15,000 | Rough estimate—refine after site selection |
| Printing (questionnaires, consent forms) | 2,000–3,000 | Rough estimate |
| Mobile data / connectivity for field data entry | 1,000–2,000 | Rough estimate |
| Respondent incentive (optional, if ethics board permits) | 0–10,000 | Institutional policy dependent—[DATA REQUIRED] |
| Software | 0 | MATLAB is typically available via university license—confirm access. |
| Miscellaneous / contingency | 5,000 | Rough estimate |

# Significance of the Study

Electric easy-bikes represent one of the most critical backbones of informal public transit in Bangladesh, providing low-cost mobility and vital livelihoods. However, the sector remains heavily dependent on conventional lead-acid (LA) batteries, which present severe inefficiencies, high replacement frequencies, and toxic end-of-life environmental risks associated with informal recycling practices. While lithium-ion alternatives—specifically Nickel Manganese Cobalt (NMC) and Lithium Iron Phosphate (LFP)—offer superior energy density and extended cycle life, their high initial capital costs, thermal sensitivity under localized climate conditions, and market uncertainty hinder widespread adoption.

This study addresses a critical knowledge gap by developing a localized, empirical, and degradation-aware techno-economic framework specifically calibrated for Bangladesh’s easy-bike sector. Unlike existing literature that often treats lithium-ion as a monolithic technology or relies on generic assumptions from other South Asian contexts, this research systematically compares LA, NMC, and LFP chemistries across integrated operational, economic, and environmental dimensions.

The significance of this work operates on three key levels:

- Academic & Methodological Contribution: It bridges field survey data with MATLAB-based computational modeling, utilizing literature-adapted degradation scenario multipliers (temperature and depth of discharge) validated against real-world operator experiences, establishing a replicable framework for informal EV assessments in developing nations.

- Industry & Market Insights: By establishing true total cost of ownership (TCO) and pairwise break-even thresholds, the study provides operators, fleet managers, and technology suppliers with clear operational parameters regarding under what exact conditions lithium substitution becomes financially viable.

- Policy & Environmental Value: The study delivers quantified policy scenario evaluations (import duties, subsidies, and financing structures) alongside a simplified lifecycle environmental assessment. This directly assists government agencies (e.g., SREDA, BERC) in formulating targeted intervention strategies to modernize informal transit while mitigating toxic heavy-metal exposure from informal lead-acid recycling.

# Expected Outcomes

This research will yield several concrete, data-driven outputs designed to clarify the feasibility of battery transition in Bangladesh's informal electric transit sector:

- Validated Comparative TCO Models: Comprehensive Total Cost of Ownership profiles and pairwise break-even matrices (LA–NMC, LA–LFP, NMC–LFP) incorporating localized grid tariffs, maintenance costs, and scrap value dynamics under varying discount rates.

- Degradation & Lifetime Sensitivity Mapping: Quantification of how seasonal temperature fluctuations and operational Depth of Discharge (DoD) impact real-world battery longevity, visualized through single-variable sensitivity tornado charts.

- Empirical Operator Baseline: A primary survey dataset from 60–80 operators across target cities, establishing benchmark operational practices, battery replacement cycles, usage profiles, and field-identified lithium chemistry adoption levels.

- Quantified Policy & Environmental Scenarios: Scenario-based evaluations assessing the threshold of capital subsidies or tax restructurings required to incentivize lithium adoption, alongside a comparative lifecycle emissions baseline (kg CO2-eq/vehicle-km) reflecting Bangladesh’s energy grid and battery end-of-life pathways.

Would you like me to draft a specific Hypotheses Formulation (H1–H8) section or adjust the work plan timeline to align directly with these outcomes?

# Target Journals (Verify Quartile Before Submission)

| Journal | Fit |
| --- | --- |
| Journal of Energy Storage (Elsevier, Q1 2024–25 SJR/JCR) | Battery tech, degradation, techno-economics |
| Sustainable Energy Technologies and Assessments (Elsevier, Q2 2025 JCR) | Energy technology assessment, economics, policy |
| World Electric Vehicle Journal (MDPI) — [DATA REQUIRED: verify quartile] | EV batteries, developing-country e-mobility |
| Energy for Sustainable Development (Elsevier) — [DATA REQUIRED: verify quartile] | Developing-country energy/transport access |
| Case Studies on Transport Policy (Elsevier) — [DATA REQUIRED: verify quartile] | Informal/paratransit transport policy |

# References (IEEE).

[1] “Experimental Study on Lead Acid Battery Driven E-Rickshaw Performance Using Capacitor Bank,” J. New Mater. Electrochem. Syst., IIETA, DOI:10.14447/jnmes.v27i1.a05.

[2] “Effect of fast charging and equalization on the reliability and cycle life of lead acid batteries,” J. Energy Storage, Elsevier, 2022.

[3] “Techno–economic and environmental impact analysis of electric two-wheeler batteries in India,” Clean Energy, Oxford Academic, vol. 8, no. 3, 2024–25.

[4] “Bi-linear capacity decay and internal resistance increase of LFP cell in electric-rickshaw application,” Elsevier (ScienceDirect), 2024–25.

[5] M. Iqbal, S. Akhter, S. Ahmad, “Battery Operated Easy Bike vs. CNG Auto Rickshaw at Sylhet City, Bangladesh,” 2013.

[6] “A Case Study on E-rickshaws in Rangpur City, Bangladesh,” J. Bangladesh Institute of Planners, vol. 12, pp. 55–64, 2019.

[7] S. Ahmad, A. Al Amin, “The rise of battery-run rickshaws in Bangladesh,” Intent Research Center, Oct. 2025.

[8] “Used EV batteries could be used for rickshaws in Bangladesh,” WMG, Univ. of Warwick, Nov. 2019.

[9] B. Battke et al., “Review and probabilistic model of lifecycle costs of stationary batteries,” Renew. Sustain. Energy Rev., vol. 25, pp. 240–250, 2013.

[10] J. Hagman et al., “Total cost of ownership and its implications for BEV diffusion,” Res. Transp. Bus. Manag., vol. 18, 2016.

[11] “Total Cost of Ownership of Electric Vehicles: A Synthesis of Critical Factors,” IET/Wiley, DOI:10.1049/tje2.70113, 2024–25.

[12] “Battery Asset Management with Cycle Life Prognosis,” arXiv:2011.14903.

[13] UNEP, “Used Lead-Acid Batteries (ULAB): Waste and Lead,” incl. Bangladesh pilot project.

[14] “Govt readies rules to pave way for electric vehicles,” The Daily Star.

[15] “EV incentives unveiled in Bangladesh as BYD eyes expansion,” electrive.com, Jun. 2026.

[16] “Battery rickshaws: From transport solution to menace on Dhaka roads,” The Business Standard, 2026 (citing a 2025 BUET fleet estimate).

[17] cmbatteries.com, sunlithenergy.com, anernstore.com, industry technical comparison pages on LFP vs. NMC cycle life and temperature sensitivity (non-peer-reviewed; retained only because it is the source of a specific LFP-vs-NMC comparison claim in the text and must be corroborated with peer-reviewed sources in Phase 1).

[18] Y. Han et al., “Optimal Operating Strategy for Battery Life Cycle Costs in EVs,” J. Appl. Math., vol. 2014, 2014.

[19] “Bayesian Monte Carlo-assisted LCA of LFP battery production under uncertainty,” Green Manuf. Open, vol. 2, no. 18, 2024.

[20] M. A. H. Mondal et al., “Future choice of technologies and co-benefits of CO₂ reduction in Bangladesh power sector,” Energy, vol. 35, no. 12, 2010.